OWNER’S MANUAL

# MODEL NUMBERS

# LRC6-00

# LRC5-00

# LRC4-00

CAUTION

For Safe Operation

Read Rules And

Instructions Carefully

## LIGHT DUTY 3-PT. MOUNTED ROTARY MOWER

## SPECIFICATIONS:

###### 11 Gauge (1/8”) One-piece Deck

40 HP Gearbox

Balanced Blade Pan with Integral Stump Jumper

Quick 1 Bolt Tailwheel Height Adjustment

Through-Deck Blade Bolt Access

Heavy Duty Laminated Tailwheel

### SAFETY INSTRUCTIONS          OPERATING INSTRUCTIONS           MAINTENANCE

TRACTOR PREPARATION         ASSEMBLY & MOUNTING               REPAIR PARTS

## CAUTION

THE FOLLOWING SAFETY PRECAUTIONS SHOULD BE THOROUGHLY            UNDERSTOOD BEFORE ATTEMPTING TO BEGIN ASSEMBLING THIS MACHINE.

1. Select an area for assembly that is clean and free

of any debris which might cause persons working

on the assembly to trip.

2. Do not lift heavy parts or assemblies. Use crane,

jack, tackle, fork trucks or other mechanical devices.

3. Preview the assembly instructions in your operator’s

manual before proceeding further.

4. If the assembly instructions call for parts or assemblies

to be blocked up, use only blocking material that is in

good condition and is capable of handling the weight

of the assembly to be blocked. Also insure that the

blocking material is on a clean, dry surface.

5. Never put hands, or any part of your body, under

blocked up assemblies if at all possible.

6. After completing assembly, thoroughly inspect the

machine to be sure that all nuts, bolts, hydraulic fittings

or any other fastened assemblies have been tightened.

7. Before operating the machine, thoroughly read the

operation section of your operator’s manual.

8. Before operating, read the maintenance section of

your operator’s manual to be sure that any parts

requiring lubrication, such as gearboxes, are full,

to avoid any possible damage.

9. Before operating equipment – if you have any

questions regarding the proper assembly or

operation, contact your dealer or representative.

## STATEMENT OF POLICY

It is the policy of Wallace Auto Parts & Services, Inc. to improve its products where it is possible and practical to do so. Wallace Auto Parts & Services, Inc. reserves the right to make changes or improvements in design and construction at any time, without incurring the obligation to make these changes on previously manufactured units.

## TABLE OF CONTENTS

SPECIFICATIONS				Cover

CHECKLIST					Page 4

SAFETY INFORMATION			Page 5

PREPARATION INSTRUCTIONS 		Page 7

PARTS LIST					Page 8

SAFETY SIGNS				Page 10

OPERATING INSTRUCTIONS		Page 18

OWNER MAINTNANCE & LUBRICATION	Page 22

TROUBLESHOOTING				Page 25

WARRANTY					Page 28

## TO THE OWNER:

Read this manual before using your rotary mower. This manual is provided to give you the necessary operating and maintenance instructions for keeping your rotary mower in top operating condition. Please read this manual thoroughly. Understand what each control is for and how to use it.

Observe all safety signs on the machine and noted throughout the manual for safe operation of the implement. Keep this manual handy for ready reference.

Like all mechanical products, it will require cleaning and upkeep. Lubricate the mower as specified.

## Use only genuine Wallace Auto Parts & Services, Inc. service parts. Substitute parts will void warranty and may not meet standards required for safe and satisfactory operation.

## RETAIL CUSTOMER’S RESPONSIBILITY

It is the retail customer and/or Operator’s responsibility to read the Operator’s Manual, to operate, lubricate, maintain, and store the product in accordance with all instructions and safety procedures.

Failure of the operator to read the Operator’s Manual is a misuse of this equipment.

It is the retail customer and/or Operator’s responsibility to inspect the product and to have any part(s) repaired or replaced when continued operation would cause damage or excessive wear to other parts or cause safety hazard.

It is the retail customer and/or Operator’s responsibility to deliver the product to the authorized Wallace Dealer, from whom it was purchased, for service or replacement of defective parts that are covered by warranty. Repairs to be submitted for warranty consideration must be made within ninety (90) days of failure.

It is the retail customer and/or Operator’s responsibility for any cost incurred by the Dealer for traveling to or hauling of the product for the purpose of performing a warranty obligation or inspection.

## CHECKLISTS

### DELIVERY CHECKLIST

Inspect the mower thoroughly after assembly to be certain it is set up set up properly. The following checklist is a reminder of points to inspect. Check off each as it is found satisfactory or after proper adjustment is made.

Check operator’s manual and familiarize the operator with all sections of it.

Check that all safety shielding is in place.

Check all bolts to be sure they are tight or adjusted properly at hinged locations.

Check that all cotter pins are properly installed.

Check PTO driveline. Make sure it is the correct length to operate mower with intended tractor.

Check all lubrication points with grease fittings have been lubricated.

Check that blades have been properly installed. Check all blade beam bolts.

Check mower attitude, after mounting on tractor.

Check that gearbox is properly serviced and seals are not leaking.

Check shear bolt for proper grade and installation.

Check front of input gearbox shaft and make sure that snap ring is properly installed.

Check that all safety signs (decals) are in place and readable.

## DAILY CHECKLIST

Check that mower is properly and securely attached to tractor.

Check that all nuts and bolts are secure and that all safety shields are in place.

It is very important that the blade carrier nut and blade bolts are checked and tightened frequently.

Check condition of blades and security of attachment.

Check that PTO driveline shields are securely locked and clears the front of the mower frame.

Adjust the tractor top link so the front of the mower is slightly above the rear. Then raise the tractor lift very slowly, making sure that the front driveshaft shield does not hit the front of the mower. If it does, damage will be done to the driveshaft shield and, if it hits hard enough, it will also damage the driveshaft itself.

#### NOTE: This type of damage is not covered under warranty, as it is totally under the control and the responsibility of the operator.

## IMPORTANT!

GEARBOX WAS NOT FILLED AT FACTORY. IT MUST BE SERVICED PRIOR TO OPERATING. FILL TO PROPER WITH LEVEL WITH 80/90 GEAR OIL. FAILURE TO SERVICE WILL RESULT IN DAMAGE TO GEARBOX.

## TO THE OWNER / OPERATOR / DEALER

All implements with moving parts are potentially hazardous. There is no substitute for a cautious, safe-minded operator who recognizes the potential hazards and follows reasonable safety practices. The manufacturer has designed this implement to be used with all its safety equipment properly attached to minimize the chance of accidents.

## BEFORE YOU START!!

Read the safety messages on the implement and shown in your manual.

Observe the rules of safety and common sense!

THE SAFETY ALERT SYMBOL IDENTIFIES IMPORTANT SAFETY WARNING MESSAGES. CAREFULLY READ EACH WARNING MESSAGE THAT FOLLOWS.

FAILURE TO UNDERSTAND AND OBEY A SAFETY WARNING, OR RECOGNIZE A SAFETY HAZARD, COULD RESULT IN AN INJURY OR DEATH TO YOU OR OTHERS AROUND YOU. THE OPERATOR IS ULTIMATELY RESPONSIBLE FOR THE SAFETY OF HIM/HERSELF, AS WELL AS OTHERS, IN THE OPERATING AREA OF THE EQUIPMENT.

UNDERSTAND SIGNAL WORDS

# A signal word – DANGER, WARNING OR CAUTION- is used with the safety-alert symbol. DANGER identifies the most serious hazards.

# DANGER and WARNING safety signs are located near the specific hazard.

# CAUTION safety signs list general precautions.

# The safety-alert symbol is also used in this manual to call attention to safety messages.

## IMPORTANT SAFETY INFORMATION!

Working with unfamiliar equipment can lead to careless injuries. Read this manual, and the manual for your tractor, before assembly or operating, to acquaint yourself with the machines. It is the implement owner’s responsibility, if this machine is used by any person other than you, is loaned or rented, to make certain that the operator, prior to operating:

- Reads and understands the operator’s manuals.

- Is instructed in safe and proper use.

The use of this equipment is subject to certain hazards, which cannot be protected against by mechanical means or product design. All operators of this equipment must read and understand this entire manual, paying particular attention to safety and operating instructions, prior to using. If there is something in this manual you do not understand, ask your supervisor, or dealer, to explain it to you.

Most accidents occur because of neglect or carelessness. Keep all helpers and bystanders’ twenty-five feet (25’) from an operating rear blade. Only properly trained people should operate this machine. It is recommended the tractor be equipped with Rollover Protection System (ROPS) and a seat belt that is used.

Always stop the tractor, set brake, shut off the tractor engine, remove the tractor key, and lower the implement to the ground before dismounting tractor. Never leave tractor or equipment running while unattended.

Please remember it is important that you read and heed the safety signs on the rear blade, and the safety rules set forth. Clean or replace all safety signs if they cannot be read clearly or understood. They are there for your safety as well as the safety of others. The safe use of this machine is strictly up to you, the operator.

## SAFETY INSTRUCTIONS (CONTINUED)

EQUIPMENT SAFETY GUIDELINES

Safety of the operator is one of the main concerns in designing and developing a new piece of equipment. Designers and manufacturers build in as many safety features as possible. However, every year many accidents occur which could have been avoided by a few seconds of thought and more careful approach to handling equipment. You, the operator, can avoid many accidents by observing the following precautions in this section. To avoid personal injury, study the following precautions and insist those working with you, or for you, follow them.

In order to provide a better view, certain photographs or illustrations in this manual may show an assembly with a safety shield removed. However equipment should never be operated in this condition. Keep all shields in place. If shield removal becomes necessary for repairs, replace the shield prior to use.

Replace any CAUTION, WARNING, DANGER or instruction safety sign that is not readable or is missing. Location of such safety signs is indicated in this booklet.

Never use alcoholic beverages or drugs, which can hinder alertness or coordination while operating this equipment. Consult your doctor about operating this machine while taking prescription medications.

Review the safety instructions with all users annually.

This equipment is dangerous to children and persons unfamiliar with its operation. The operator should be a responsible adult familiar with farm machinery and trained in this equipment’s operations. Do not allow persons to operate or assemble this unit until they have read this manual and have developed a thorough understanding of the safety precautions and how it works.

To prevent injury or death, use a tractor equipped with a Roll-Over Protective System (ROPS). Do not  paint over, remove or deface any safety signs or warning signs on your equipment. Observe all safety signs and practice the instruction on them.

Never exceed the limits of a piece of machinery. If its ability to do a job, or to do so safely, is in question – Don’t try it.

Do not  modify the equipment in any way. Unauthorized modification may impair the function and/or safety and could affect the life of the equipment.

In addition to the design and configuration of this implement, including Safety Signs and Safety Equipment, hazard control and accident prevention are dependant upon the awareness, concern, prudence, and proper training of personnel involved in the operation, transport, maintenance, and storage of the machine. Refer also to safety messages and operation instruction in each of the appropriate sections of the tractor and mower manuals. Pay close attention to the safety signs attached to the tractor and mower.

Keep safety signs clean and legible at all times.

Replace safety signs that are missing or become illegible.

Replaced parts that displayed a safety sign should also display the current sign.

Safety signs are available from your Distributor or Dealer Parts Department or the Factory.

Safety is a primary concern in the design and manufacture of our products. Unfortunately, our efforts to provide safe equipment can be wiped out by a single careless act of an operator.

In addition to the design and configuration of equipment, hazard control and accident prevention are dependent upon the awareness, concern, prudence and proper training of personnel involved in the operation, transport, maintenance and storage of equipment.

It has been said , “the best safety device is an informed, careful operator.” We ask you to be that kind of an operator. It is the operator’s responsibility to read and understand ALL Safety and Operating instructions in the manual and to follow them. Accidents can be avoided.

Working with unfamiliar equipment can lead to careless injuries. Read this manual, and the manual for your tractor, before assembly or operating, to acquaint yourself with the machines. It is the implement owner’s responsibility, if this machine is used by any person other than you, is loaned or rented, to make certain that the operator, prior to operating:

- Reads and understands the operator’s manuals.

- Is instructed in safe and proper use.

Know your controls and how to stop the tractor, engine, and mower quickly in an emergency. Read this manual and the one provided with your tractor.

Train all new personnel and review instructions frequently with existing workers. A person who has not read and understood all operating and safety instructions is not qualified to operate the machine. An untrained operator exposes himself and bystanders to possible serious injury or death.

Do not allow children to operate this machine.

### PREPARATION

Never operate the tractor or mower until you have read and completely understand this manual, the Tractor Operator’s Manual, and each of the Safety Messages found on the safety signs on the tractor and mower.

Personal protection equipment including hard hat, safety glasses, safety shoes, and gloves are recommended during assembly, installation, operation, adjustment, maintaining, repairing, removal, or moving the implement. Do not allow long hair, loose clothes or jewelry to be around moving parts.

# PROLONGED EXPOSURE TO LOUD NOISE MAY CAUSE PERMANENT HEARING LOSS!

Tractors with or without Mowers attached can often be noisy enough to cause permanent, partial hearing loss. We recommend that you wear hearing protection on a full-time basis if the noise in the Operator’s position exceeds 80db. Noise over 85db on a long-term basis can cause severe hearing loss. Noise over 90db adjacent to the operator over a long-term basis may cause permanent, total hearing loss. NOTE: hearing loss from loud noise (from tractors, chain saws, radios, and other such sources close to the ear) is cumulative over a lifetime without hope of natural recovery.

Operate the mower only with a tractor equipped with an approved Roll-Over Protection System (ROPS).

Always wear your seatbelt on tractors equipped with ROPS. Serious injury or even death could result from falling off the tractor , particularly during a turn over when the operator could be pinned under the ROPS or the tractor.

Clear area to be cut of stones, branches or other debris that might be thrown, causing injury or damage.

Operate only in daylight or good artificial light.

Ensure mower is properly mounted, adjusted and in good operating condition.

Make sure driveline spring-activated locking pin or balls operate freely and are seated firmly in tractor PTO stub shaft groove.

Ensure that all safety shielding and safety signs are properly installed and in good condition.

### Tailwheel Assembly

### Tailwheel A-Frame

- Cotter Pin

- Washer

- Tailwheel Arm Assembly

- Nylon Locknut

- Washer

- Tailwheel Bolt

- Tailwheel

### Top Link Clevis Assembly

- Clevis Pivot Bolt

- Washer

- Nut

### Clevis

- Hitch Mount

- Lift Strap

### Light Duty Rotary Cutter

### Toplink Clevis			9.	Tailwheel Adjustment Bracket

- Hitchmount			10.	Tailwheel A-Frame

- Lift Pin Bracket		11.	Tailwheel Assembly

- Lift Pin			12.	Gearbox

- Danger Label			13.	Lift Strap

- Deck				14.	40 HP Safety Label

- Skid Plate

- PTO Safety Label

### STARTING AND STOPPING SAFETY

Mower operating power is supplied from tractor PTO. Refer to your tractor manual for PTO engagement and disengagement instructions. Always operate PTO at 540rpm. Know how to stop tractor and mower quickly in case of an emergency.

When engaging PTO, the engine RPM should always be low. Once engaged and ready to start cutting, raise

PTO speed to 540rpm and maintain throughout cutting operation.

Check the tractor master shield over PTO stub shaft. Make sure it is in good condition and fastened securely to the tractor. Purchase a new shield if old shield is damaged or missing. (You may have to use a tractor salvage yard for replacement parts on older tractors.)

All tractors that are not equipped with a “live” takeoff (PTO) need to be equipped with an over-running PTO clutch type driveline. These are available as optional equipment.

NOTE: The addition of a separate over-running PTO clutch attachment will change the length of the PTO driveline. Pay extra attention to the instructions on the PTO driveline installation.

Separate over-running clutch attachments will increase the length of the driveline and may cause the front knuckle joint to operate beyond the cover of the tractor master shield. This is NOT recommended and will prevent a dangerous operating condition.

### OPERATIONAL SAFETY

The use of this equipment is subject to certain hazards, which cannot be protected against by mechanical means or product design. All operators of this equipment must read and understand this entire manual, paying particular attention to safety and operating instructions prior to using. If there is something in this manual you do not understand, ask your supervisor, or your dealer, to explain it to you.

Most accidents occur because of neglect or carelessness. Keep all helpers and bystanders at least several hundred feet away from an operating rotary mower. Only properly trained people should operate this machine.

When this machine is operated in populated areas or in other areas where thrown objects could injure persons or property, standard equipment safety chain shielding (which is designed to reduce the possibility of thrown objects) must be installed. If this machine is not equipped with full chain shielding, operation must be stopped when anyone comes within several hundred feet.

The majority of accidents involve entanglement on the driveline, injury of bystanders by objects thrown by the rotating blades, and operators being knocked off the tractor by low hanging limbs and then being run over by the mower. Accidents are most likely to occur with machines that are loaned or rented to someone who has not read the owner’s manual and is not familiar with a rotary mower.

Always stop the tractor, set brake, shut off tractor engine, remove the ignition key, lower implement to the ground and allow cutter blades to come to a complete stop before dismounting tractor. Never leave equipment unattended with the tractor running.

Never place hands or feet under mower with tractor engine running or before you are sure all motion has stopped. Stay clear of all moving parts.

Do not reach or place yourself under equipment until it is blocked securely.

Take all possible precautions when leaving unit unattended: disengage PTO, set parking brake, stop engine and remove key from ignition. Park in level area.

Do not allow riders on the rotary mower or tractor at any time. There is no safe place for any riders.

Disengage power takeoff (PTO) and place transmission into neutral before attempting to start engine.

Do not operate unless all personnel, livestock, and pets are several hundred feet away to prevent injury by thrown objects. Never direct discharge toward anyone.

Install and secure all guards and shields before starting or operating.

Keep hands, feet, hair and clothing away from moving parts.

This rotary mower is designed for use only on tractors with 540rpm takeoff.

Never operate tractor or mower under trees with low hanging limbs. Operators can be knocked off the tractor and be run over by the rotating blades.

The rotating parts of this machine have been designed and tested for rugged use. However, they could fail upon impact with heavy, solid objects such as steel guardrails, post, and concrete abutments. Such an impact could cause the broken objects to be thrown outward at very high velocities. To reduce the possibility of property-damage, serious injury, or even death, Never allow the cutting blades to contact such obstacles.

Frequently check mower blades. They should be sharp, free of nicks and cracks and securely fastened.

Many varied objects, such as wire, cable, rope, or chains, can become entangled in the operating parts of the mower head. These items could then swing outside the housing at greater velocities than the blades. Such a situation is extremely hazardous. Inspect the cutting area for objects before mowing. Remove any objects of this nature from mowing area. Never allow the cutting blades to contact such items.

Pick up all rocks and other debris before mowing. Enter new areas carefully. Cut material higher at first, to allow cutter to clear hidden objects. Never assume an area is clear. Always check!

Stop mower and tractor immediately upon striking an obstruction. Turn engine off, remove key, inspect and repair any damage before resuming operation.

The chain guards, bands, flaps, driveline shields, and gearbox shields should be used and maintained in good working condition. They should be inspected carefully, at least daily, for missing or broken cable, chain links, shields, or guards. Missing, broken, or worn items must be replaced at once to reduce the possibility of injury from thrown objects or entanglement.

Stay alert for holes, rocks and roots in the terrain and other hidden hazards. Keep away from drop-offs.

Use extreme care and maintain minimum ground speed when transporting on hillside, over rough ground and when operating close to ditches or fences. Be careful when turning sharp corners.

Reduce speed on slopes and sharp turns to minimize tipping or loss of control. Be careful when changing directions on slopes. Do not start or stop suddenly on slopes. Avoid operation on steep slopes.

When using a unit, a minimum of 20% of the tractor and equipment weight must be on tractor front wheels. Without this weight, the tractor could tip over, causing personal injury or death. The weight may be attained with a front-end loader, front wheel weights, ballast in tires of front tractor weights. When attaining a minimum of 20% of tractor and equipment weight on front wheels, you must not exceed the ROPS weight certification. Weigh the tractor and equipment. Do not guess or estimate!

Inspect the entire machine periodically as indicated in the Maintenance Section of this manual. Look for loose fasteners, worn or broken parts, pinched hydraulic hoses, and leaky or loose fittings. Make sure all pins have cotter pins and washers. Serious injury may occur from not maintaining this machine in good working order.

Be careful when operating tractor and mower on uneven ground to avoid upsetting.

In extremely uneven terrain, rear wheel weights, front tractor weight, and/or tire ballast should be used to improve stability.

Pass diagonally through sharp dips and avoid sharp drops to prevent “hanging up” tractor and cutter. Practice will improve your skills in maneuvering rough terrain.

Avoid sudden starts and stops while traveling up or downhill.

Always cut down slopes; never across the face. Avoid operation on steep slopes. Slow down on sharp turns and slopes to prevent tipping and/or loss of control.

## TRANSPORT SAFETY

Comply with state and local laws governing highway safety and movement of farm machinery on public roads.

The use of flashing amber lights is acceptable in most localities. However, some localities prohibit their use. Local laws should be checked for all highway lighting and marking requirements.

When driving the tractor and equipment on the road or highway under 20mph (32kph) at night or during the day, use flashing amber warning lights and slow moving vehicle (SMV) identification emblem.

Always be sure the implement is in the proper raised position for transport.

Reduce speed when transporting mounted implements to avoid bouncing and momentary loss of steering control.

Plan your route to avoid heavy traffic.

Always install transport locks, pins or brackets before transporting.

Do not drink and drive!

Watch for traffic when operating near or crossing roadway.

Turn curves or go up or down hills only at a low speed and at a gradual steering angle. Make certain that at least 20 % of the tractor’s weight is on the front wheels to maintain safe steering. Slow down on rough or uneven surfaces, and loose gravel.

Use extreme care and maintain minimum ground speed when transporting on hillside, over ground and when operating close to ditches or fences. Be careful when turning sharp corners.

Never allow riders on either tractor or mower. Falling off can kill!

Be a safe and courteous driver. Always yield to oncoming traffic in all situations, including narrow bridges, intersections, etc.

Do not exceed 20mph (32kph). Reduce speed on rough roads and surfaces.

Use hardened hitch pins with retainers when attaching to pull-type machines.

Use safety chains to prevent unexpected separation with pull-type models.

## MAINTENANCE SAFETY

Good maintenance is your responsibility. Poor maintenance is an invitation to trouble.

Follow good shop practices.

-Keep service area clean and dry.

-Be sure electrical outlets and tools are properly grounded.

-Use adequate light for the job at hand.

Make sure there is plenty of ventilation. Never operate the engine of the towing vehicle in a closed building. The exhaust fumes may cause asphyxiation!

Be certain that all moving parts have come to a complete stop before attempting to perform maintenance.

Keep all persons away from operator control area while performing adjustments, service, or maintenance.

Always use personal protection devices such as eye, hand and hearing protectors, when performing any service or maintenance.

Openings in the skin and minor cuts are susceptible to infection from hydraulic fluid. If injured by escaping hydraulic fluid, see doctor at once. Gangrene and/or death can result. Without immediate medical treatment, serious infection and reactions can occur.

After servicing, be sure all tools, parts and service equipment are removed.

Check to ensure all safety signs are installed and are in good condition.

Do not allow grease or oil to build up on deck or platform.

Never replace hex bolts with less than grade five bolts unless otherwise specified, i.e. shear bolts.

Where replacement parts are necessary for periodic maintenance and servicing, genuine factory replacement part must be used to restore your equipment to original specifications. The manufacturer will not claim responsibility for use of unapproved parts and/or accessories and other damages as a result of their use.

If the equipment has been altered in any way from original design, the manufacturer does not accept any liability for injury or warranty.

A fire extinguisher and first aid kit should be kept readily accessible while performing maintenance on this equipment.

## STORAGE SAFETY

Following operation, or when unhooking, stop tractor, set the brakes, disengage the PTO, shut off the engine and remove the ignition keys.

Store the unit in an area away from human activity.

Do not park equipment where it will be exposed to livestock for long periods of time. Damage and livestock injury could result.

Do not permit children to play on or around the stored unit.

Make sure all parked machines are on a hard, level surface and engage all safety devices.

If blocking is used, make sure it is solid and secure before leaving area.

INSTRUCTIONS

### TRACTOR REQUIREMENTS AND PREPARATION

The light duty mowers require a category 1, 3-pt. hitch tractor of at least 20hp for 4’ cutter, 25hp for 5’ cutter, and 30hp for 6’ cutter or up to 40hp maximum for all light duty cutters. Tractor also should be equipped with a 540 rpm PTO in good working condition.

It is recommended that only tractors with wide front axles be used with this cutter. Tricycle front wheel arrangements are inherently unstable and tractor rollover accidents are more likely to occur.

The tractor also needs lift arm stabilizers bars or sway blocks to control side movement of cutter.

Check the tractor’s 3-pt. hydraulic lift system. Refer to your tractor operator’s manual or dealer for any adjustments necessary to put the hydraulic system in good working order.

Be sure the tires and rims are in good condition. Inflate tires to proper recommended air pressure.

All tractors that are not equipped with a “live” power takeoff (PTO) need to be equipped with an over-running PTO clutch attachment. A special driveline assembly with an over-running clutch incorporated is available as an option.

Check the tractor master shield over the PTO stub shaft. Make sure it is in good condition and fastened securely to the tractor. Purchase a new shield if old shield is damaged or missing.

It is recommended that a roll over protection structure and seat belt be installed on all tractors.

Contact your local dealer for a ROPS for your tractor.

The operator is responsible for the safe operation of this mower. The operator must be properly trained.

Operators should be familiar with the mower and tractor and all safety practices before starting operation.

Read and understand all safety rules and safety signs.

The rotary mower is designed for grass and weed mowing and/or shredding.

The recommended mowing speed for most conditions is from two (2) to five (5) MPH.

- Raise and lower the mower to determine position with greatest distance between the PTO shaft and gearbox input shaft. Shut tractor off, leaving mower in position of greatest distance between shafts. SECURELY BLOCK MOWER IN POSITION.

- Hold driveline sections parallel to each other and check for minimum 6” (15cm) overlap. If driveline has been marked for cutting, overlap will be the distance between the two marks. If driveline has less than minimum overlap, do not use. Contact authorized dealer.

- If driveline must be cut to shorter length, clamp driveline in well-padded vise to prevent damage to the shield. Cut off shield where marked. Using cut-off section of shield as a guide, cut shaft the same amount.

- Repeat the procedure to the other half of the driveline. Remove all burrs and cuttings.

- Apply multi-purpose grease to the inside of outer (female) section of the driveline. Assemble driveline and install on tractor and mower. Pull on each driveline section to be sure yokes lock into place. Make certain driveline shielding is in place and is in good condition.

- Carefully raise and lower the mower and check to be sure that the PTO assembly does not jam. If it does, cut equal amounts from each half of the PTO assembly.

Adjust the tailwheel and lift arms so the mower is level. Then raise the tractor lift very slowly, making sure that the front drive shaft shield does not hit the front of the mower. If it does, damage will be done to the drive shaft shield and, if interference is bad enough, it will also damage the drive shaft itself.

NOTE: This type of damage is not covered under warranty, as it is totally under the control and the responsibility of the operator.

Use the lift control limiting stop on the tractor control lever to limit the upward travel of the lever so the lift cannot be raised high enough to cause contact between the drive shaft shield and front shielding.

Check the swinging drawbar of the tractor and make sure the PTO driveline assembly will not contact. The swinging drawbar can be moved forward on some tractors or it can be removed.

If the PTO driveline assembly contacts the swinging drawbar, damage will occur to the driveshaft shield and possibly the driveshaft itself. (NOT covered under warranty.)

## ATTACHMENT

The mower is shipped partially assembled. Assembly will be easier if components are aligned and loosely assembled before tightening hardware.

Select a suitable working area. Refer to illustrations accompanying text, parts list and exploded view drawings.

Complete checklist when assembly is complete.

Position mower flat and cut nylon straps that are holding all loose parts to the mower.

Rotate the “A” frame hitch forward and connect lift arms and toplink.

Attach the two linkage lift straps to the mower deck (behind the gearbox) and to the end of the slack link on the “A” frame. These bolted connections are moveable joints – so tighten nuts only until excess looseness is gone.

Most swinging drawbars will have to be moved to a forward position or removed. Check the tractor’s swinging drawbar for interference with the front of the mower before attempting to lift the mower with the 3-pt. hitch.

Tractor lift arm stabilizer bars or sway blocks must be used to control side movement of the cutter. Do not connect the PTO driveline at this time.

Check your lift arm hydraulic controls. Be sure the hydraulic 3-pt. hitch control is in the float position and the draft control is turned off.

Adjust lower lift arm(s) to level mower right to left. Refer to the tractor operator’s manual for instructions.

Cutting height is controlled with tractor 3-point arms, and rear tailwheel adjustment.

## PTO DRIVELINE INSTALLATION

Spray WD-40 into yoke and wipe. This should remove some of the paint and make it easier to slide the yoke onto the input shaft of the gearbox.

Remove shear-bolt and retaining ring from gearbox input shaft.

Grease the input shaft of the gearbox before installing the PTO shaft. This reduces the chance of the PTO shaft yoke from galling to the input shaft if the shear pin should break.

Remove rear drive shield from driveline. To remove, turn each plastic clip ¼ turn and then lift out. Then slide rear shield so entire joint assembly is exposed.

To prevent seal damage carefully push driveline onto gearbox input shaft until it contacts gearbox housing.

Install retaining ring and then pull driveline ahead.

NOTE: A grade 2 bolt must be used for the shear bolt to provide gearbox protection.

Align the holes in the driveline yoke and gearbox input shaft and install and tighten shear bolt (6) and nut (5).

Install rear drive shield to driveline.

Lubricate rear driveline half and install front driveline half.

In some cases it will be necessary to shorten the PTO assembly to match your particular tractor, the following procedure should be used.

- Raise the tractor 3-pt. hitch so the input shaft of the mower gearbox is in line with the PTO shaft on the tractor. Shut down tractor, leaving mower in position of shortest distance between shafts. Securely block mower into position.

- Pull driveline apart. Attach outer (female) section to tractor PTO shaft. Pull on driveline section to be sure that yoke locks are in place.

- Hold driveline sections parallel to each other to determine if too long. Each section should end approximately 3” (76mm) short of reaching universal joint shield on opposite section. If too long, measure 3” (76mm) back from universal joint shield and mark on opposite section. Do this for both sections. Do not cut until steps 4 and 5 are checked.

## OPERATING INSTRUCTIONS

### GEARBOX OIL FILL

NOTE: The gearbox is shipped WITHOUT lubricant, so it will be necessary to fill it before use. With the gearbox in level position, fill to the side inspection hole with a quality 90 WT. Gear oil. Check often and add lubricant if necessary.

## GENERAL SAFETY

Only qualified people familiar with this manual should operate this machine. Operator should were hard hat, safety glasses, and safety shoes. It is recommended that your tractor, be equipped with ROPS and seat belt be used. Before beginning operation, clear work area of any objects that may be picked up and thrown. Check area for ditches, stumps, holes, or other obstacles that could upset or damage cutter. Always turn off tractor engine, set parking brake, lower cutter to the ground and allow mower blades to come to a complete stop before dismounting tractor.

Safety is our primary concern in the design and manufacture of our products. Unfortunately, our efforts to provide safe equipment can be wiped out by one single careless act of an operator.

It has been said, “the best safety device is an informed, careful operator”. We ask you to be that kind of an operator.

The designed and tested safety of this machine depends on it being operated within the limitations as explained in this manual. Be familiar with and follow all safety rules in this manual, on the mower and on the tractor.

The safe operation of this machine is the responsibility of the owner/operator. The operator should be familiar with the mower and tractor and all safety practices before starting operation.

Do not operate the PTO at other than the rated 540RPM. Excessive speeds can cause breakage, thrown objects, and potential injury.

### ADJUSTING FOR WORK

The mower should be operated at the highest position, which will give desired cutting results. This will help prevent the blades from striking the ground, reducing blade wear and undue strain on the machine. For best results always tilt the mower slightly lower in the front. This tilt decreases horsepower requirements. When fine shredding is desired, adjust mower deck level or slightly lower. This will keep foliage under mower until thoroughly shredded. More power is required for shredding.

The cutting height is adjustable from 1 – 5 inches. To adjust, proceed as follows:

- Raise mower off ground to desired cutting height using tractor 3-pt. lift. Turn off tractor engine, disengage PTO, set parking brake, and remove ignition key.

- SECURELY BLOCK MOWER.

- Remove bolt from height adjusting bracket.

- Adjust tailwheel to desired position. Secure with bolt, lock washer and nut.

- Lower mower to work position.

- Adjust tractor 3-point hitch top link so mower deck is level when the mower is raised from cutting position. The linkage clevis should have a small amount of slack when the mower is lowered to cutting position to allow mower deck to float during operation.

## PRE-OPERATION CHECKLIST (OWNER RESPONSIBILITY)

Review and follow all safety rules and safety signs in this manual.

Check that mower is properly and securely attached to tractor.

Make sure driveline spring-activated locking pin or locking balls operate freely and are seated firmly in tractor PTO spline groove.

Lubricate all grease fitting locations. Make sure PTO shaft slip-joint is lubricated.

Check to be sure that gear lube runs out the small check plug on the side of gearbox.

Check that all hardware is properly installed.

Check to ensure that blades are sharp and secure and cutting edge is positioned to lead in a counterclockwise rotation.

Check that all shields and guards are properly installed and are in good condition.

Check cutting height, front to rear attitude and top link adjustment.

Place tractor PTO and transmission in neutral before starting engine.

Set tractor PTO gear select lever for 540rpm operation.

Inspect area to be cut and remove stones, branches or other hard objects that might be thrown, causing injury or damage.

##### IMPORTANT

Vibration tends to loosen bolts during operation. All hardware should be checked regularly to maintain proper torque. It is a good practice to check mower before each operation to ensure all hardware is secure.

### OPERATING TECHNIQUE

Power for operating mower is supplied by tractor PTO. Operate PTO at 540rpm gear range. Know how to stop tractor and mower quickly in case of an emergency.

Engage PTO at a low engine rpm to minimize stress on the drive system and gearbox. Place tractor in gear and proceed forward. Advance tractor throttle to 540 PTO rpm. Tractor forward speed should be controlled by gear selection, not engine speed. For maximum cutting efficiency, forward speed should allow mower to maintain a constant, maximum blade speed. If mower stalls or tractor engine bogs, disengage PTO. Before re-engaging PTO, position mower in a cut area and reduce tractor throttle to idle. If mower continuously stalls, select lower tractor gear and/or increase cutting height.

Blade rotation is counterclockwise causing cut material to discharge to the left side of mower forming windrow. To shred and dissipate this windrow, tractor should be driven in a counterclockwise direction throwing material to the uncut area. The windrowed material will be re-cut on each new pass. Tractor tire tracks are more efficiently when tractor is traveling in a clockwise direction especially when cutting height is high. Larger brush should be cut in the center of mower. Always cut up and down the face of slopes, never across.

Proper ground speed will depend upon the terrain, the height, type and density of material to be cut.

Gearbox protection is provided by a shear-bolt. The shear-bolt will shear when excessive torsion loads are encountered.

## OPERATING TECHNIQUE (CONTINUED)

Normally, ground speed will range from two to five MPH. Tall dense material should be cut at a low speed; thin medium-height material can be cut at faster ground speed.

When engaging the PTO, the engine speed should always be low. Once the PTO is engaged and ready to start cutting, raise the PTO speed to 540rpm and maintain throughout mowing operation. ALWAYS operate the PTO at 540rpm. This is necessary to maintain proper blade speed to obtain a clean cut.

Under certain conditions, tractor tires may roll some grass down and prevent it from being cut at the same  height as the surrounding area. When this occurs, reduce your ground speed, but maintain PTO at 540rpm.

The lower ground speed will permit grass to partially rebound.

Under some conditions, grass will not rebound enough to be cut evenly. In general, lower heights give a more even cut with fewer tendencies to leave tire tracks. However, it is better to cut grass frequently rather than too short. Short grass deteriorates rapidly in hot weather and invites weed growth during growing seasons. Follow local recommendations for the suitable cutting height in your area.

### OPERATING TIPS

# SHREDDING

For shredding, it is better to set the mower lower at the rear. How much lower depends on the material to be shredded. Determine the best setting for your situation by experimenting. Inspect area to be cut and remove stones branches, or other hard objects that might be thrown, causing injury or damage.

Extremely tall material should be cut twice. Set cutter at a higher height for the first pass. Then cut at desired height at 90 degrees to the first pass. Remember that sharp blades produce cleaner cuts and require less power. Analyze area to be cut to determine the best procedure. Consider height and type of grass and terrain type, hilly, level or rough. Plan your mowing pattern to travel straight- forward whenever possible. Mow clockwise around fields when necessary to minimize streaking on corners.

# UNEVEN TERRAIN

Do not operate on steep slopes.

Do not stop, start or change directions suddenly on slopes.

Use extreme care and reduce ground speed on slopes and rough terrain.

Watch for hidden hazards on the terrain during operation.

Pass diagonally through sharp dips and avoid sharp drops to prevent “hanging up” tractor or mower. Practice will improve your skills in maneuvering rough terrain.

### BLADES

The blades will last a long time cutting grass, but will wear rapidly when contacting the ground. If they are clipping into the ground, they will have to be sharpened frequently.

The cutting action of the blade will be much cleaner, create less shock loads and require less power if the blades are kept sharp. That is with a flat or blunt edge that is not over 1/32” thick. Putting a knife-sharp edge (where both sides come to a sharp point) on the blades will not help, as a sharp edge will round off very quickly and doing so will reduce the total life of the blades

### REMOVING MOWER FROM TRACTOR

Disengage tractor PTO.

Disconnect driveline from tractor PTO.

Collapse driveline as far as possible and store it to prevent ground contact. Place blocks under mower side-skids. Lower mower onto blocks, disconnect mower from tractor 3-point hitch, and carefully drive tractor away from mower.

## OPTIONS

When this machine is operated in populated areas, along roads, or in other areas where thrown objects could injure persons or property, standard equipment safety chain shielding (which is designed to reduce the possibility of thrown objects) must be installed.

Chain guards are not mandatory for agricultural use only (i.e. – out in fields where people, livestock, or property that could be damaged are not present). If this machine is not equipped with safety with chain shielding, operation must be stopped when anyone comes within several hundred feet.

All tractors that are not equipped with a “live” power takeoff (PTO) need to be equipped with an over-running PTO clutch driveline.

Use of a standard PTO driveline that does not have an over-running clutch on these tractors, will allow the mower blade assembly to act as a flywheel. This can cause damage to the tractor’s internal PTO drive train and can make stopping the tractor’s forward travel difficult.

There are PTO over-running clutch attachments available. However, they will increase the length of the driveline and may cause the front knuckle joint to operate beyond the cover of the tractor’s master PTO shield. This is NOT recommended and a dangerous operating condition will be present.

## OWNER SERVICE

The information in this section is written for operators who posses basic mechanical skills. Should you need help, contact a qualified service technician or the factory. For your protection, read and follow all safety information in this manual.

Lower mower to ground or block securely, turn tractor engine off, remove key and disconnect mower driveline from tractor PTO before performing any service or maintenance.

Before working underneath, raise 3-pt. hitch to highest position and block securely. Hydraulic system leak-down and failure of mechanical or hydraulic system can cause equipment to drop.

Keep all persons away from operator control area while performing adjustments, service or maintenance.

### ROUTINE MAINTENANCE

DAILY CHECKS:

- Check that all bolts, nuts, and screws are tight. Checking the bolts and nuts on the blade beam assembly is particularly important.

- Check daily the level of the gearbox oil and top up to the correct level. Check for gearbox oil leaks. It should be noted that no warranty claim can be submitted on a gearbox that has run dry. It is essential that the gearbox is kept correctly filled with gearbox oil.

- Grease the PTO shaft daily

- Check the wear on the blades. Sharpen them routinely with an angle grinder or replace when worn down too far. You should keep at least two sets of blades, bolts, and nuts as spares for your mower.

AT THE END OF YOUR CUTTING SEASON:

- Drain and change the oil in your gearbox.

- Check and replace, where necessary, blades, bolts, nuts, and bushings, on the machine.

- Clean machine and touch up any rust spots that may have appeared.

- Replace any safety signs if damaged.

- Store mower in clean dry location.

### LUBRICATION INFORMATION

The accompanying chart gives the frequency of lubrication in operating hours, based on normal conditions. Severe or unusual conditions may require more frequent lubrication.

Do not let excess grease or oil collect on or around parts, particularly when operating in sandy areas.

Use 80/90-gear oil in gearbox.

Use lithium grease of NO.2 consistency with a MOLY (molybdenum disulfide) additive for all locations.

Be sure to clean fittings thoroughly before attaching grease gun. When applied according to the lubrication chart, one good pump of most guns will be sufficient. Do not over grease.

Daily lubrication of the PTO slip joint is necessary. Failure to maintain proper lubrication can result in damage to U-joints, gearbox, tractor PTO and /or mower driveline.

### BLADE SERVICING

# BLADE REMOVAL

Disconnect driveline from tractor PTO. It will be necessary to gain access to the bottom side of mower for blade removal. Raise mower and block securely. Align blade bolt with access hole in mower deck if so equipped. Remove nut and lock washer then carefully drive bolt out of crossbar.

# IMPORTANT

If blade bolt is seized in blade beam, extreme force will be required to remove it, stabilize blade assembly to prevent gearbox damage.

Rotate blade assembly and repeat for opposite blade.

Always replace or sharpen both blades at the same time.

Inspect blade bolts for nicks or gouges; replace if any are found. Insert bolt through blade. Blade should swivel on bolt, if it does not, determine the cause and correct.

Align blade bolt with access hole in mower deck. Apply a liberal coating of thread anti-seize compound to blade bolt and blade-pan hole. Make sure blade is offset away from mower.

# IMPORTANT

Blade assembly rotation is counterclockwise when looking down onto mower. Be sure to install blade cutting edge to lead in counterclockwise rotation.

# BLADE SHARPENING

# IMPORTANT

When sharpening blades, grind each blade the same amount to maintain balance. Replace blades in pairs. Unbalanced blades will cause excessive vibration, which can damage gearbox bearings. Vibration may also cause structural cracks to mower.

Sharpen both blades at the same time to maintain balance. Follow original sharpening pattern. Do not sharpen blade to a razor edge, but leave at least 1/16” blunt edge. Do not sharpen backside of blade.

### SHEAR BOLT REPLACEMENT

IMPORTANT

Always use approved ½” NC x 3” grade 2 shear-bolt as a replacement part. Using a hardened bolt or shear pin may result in damage to driveline, gearbox, or main frame.

Remove driveline shield bell.

Make sure the input shaft of the gearbox is greased. This reduces the chance of the PTO shaft yoke from galling to the input gearbox shaft if the shear-bolt should break.

Rotate driveline to align holes in yoke and shaft. Install shear-bolt and secure with lock nut. Replace driveline shield bell.

FAILURE TO USE CORRECT SIZE SHEAR-BOLT MAY CAUSE PERSONAL INJURY OR DEATH!

### FAILURE TO INSTALL RETAINING CLIP WILL ALLOW DRIVELINE TO SWING FREELY IF BOLT IS SHEARED CAUSING POSSIBLE INJURY OR DEATH!

### REPAIR OF OPTIONAL CHAIN SHIELDING

Inspect chain shielding each day of operation and replace any broken or missing chains as required.

### BLADE PAN & GEARBOX REMOVAL

- Raise mower using tractor 3-point lift. Turn off tractor engine.

- Block mower SECURELY into position.

DO NOT GET UNDER MOWER UNLESS IT IS SECURELY BLOCKED IN POSITION. ACCIDENTAL FALL COULD CAUSE SERIOUS INJURY OR DEATH.

- Remove cotter pin and blade holder retaining nut. Wear heavy gloves to protect hands from sharp edges.

- Grasp blade holder and pull off shaft. If necessary, align blade bolt with access hole in top of deck and drive off with hammer and pipe. Care should be taken not to damage threads on blade bolt.

- Press all three tabs and slide yoke shield back.

- Remove bolt and retaining clip. Slide yoke off gearbox input shaft.

- Remove nuts securing gearbox to deck. Remove gearbox.

### BLADE PAN & GEARBOX INSTALLATION

1.   Wear heavy work gloves to protect hands from sharp edges.

2.   Raise mower using tractor 3-point lift. Turn off tractor engine.

3.   Block mower SECURELY into position.

DO NOT GET UNDER MOWER UNLESS IT IS SECURELY BLOCKED IN POSITION. ACCIDENTAL FALL COULD CAUSE SERIOUS INJURY OR DEATH.

4.   Attach gearbox to mower deck using bolts and nuts.

5.   Install blade holder onto gearbox securing with castellated nut and flat washer. Torque nut to 350ft./lbs. 6.   Install cotter pin to retain nut. It may be necessary to rotate nut to install cotter pin.

7.   Slide driveline yoke onto gearbox input shaft. Install retaining clip and shear bolt.

FAILURE TO INSTALL RETAINING CLIP WILL ALLOW DRIVELINE TO SWING FREELY IF BOLT IS SHEARED CAUSING POSSIBLE INJURY OR DEATH!

8.   Lock driveline yoke shield securely into place.

9.   Fill gearbox with oil to proper level.

KEEP CHILDREN AWAY FROM DANGER ALL DAY, EVERY DAY.

EQUIP TRACTORS WITH ROLLOVER PROTECTION (ROPS) AND KEEP ALL MACHINERY GUARDS IN PLACE.

PLEASE WORK, DRIVE, PLAY, AND LIVE EACH DAY WITH CARE AND CONCERN FOR YOUR SAFETY AND THAT OF YOUR FAMILY AND FELLOW CITIZENS.

## TROUBLESHOOTING GUIDE

### Problem                                 Possible cause                             Possible Remedy

Leaves a streak of uncut          1.  Mower not level, side to side.         Level 3-pt. Hitch linkage on tractor.

or partially cut grass.

2.  Blades dull or bent.                        Sharpen or replace blades.

3.  Carrier RPM too low.                     Use correct PTO speed.

4.  Field conditions are so wet that      Too wet to mow. Stop operation

the tractor tire is pushing grass        and wait until it is drier.

into the mud.

5.  Ground speed too fast.                     Reduce ground speed by shifting

to a lower gear.

6.  Blades locked back.                         Free blades.

7.  Blades riding up due to blade          Replace blade bolts.

Bolt wear or loose bolts.

8.  Grass is down from previous            Mow in only one direction.

Weather conditions.

9.  Possible build up of material            Clean mower.

under mower.

# Grass cut lower in center          1.  Height of mower lower at rear         Adjust mower height and attitude

of swath than at edge.                     or at front.                                        So that mower rear and front are

within ½” of same height.

(See instructions.)

Material discharges from          1.  Material too high and too much       Reduce ground speed but maintain

mower unevenly; bunches              material.                                            540rpm at tractor PTO, or make

of material along swath                                                                             two passes over material. Raise

mower for the first pass and lower

to desired height for the second

and cut at 90` to the first pass.

Raise rear of mower high enough

To permit material to discharge,

but not so high that conditions

listed above occur.

2.  Grass wet.                                        Allow grass to dry before mowing.

Slow ground speed of tractor but

keep engine running at full PTO

rpm. Cutting lower will help.

3. Rear of cutter too low, trapping        Adjust mower height and attitude.

material under mower.                       (See instructions.)

Gearbox overheating.                 1.  Low on lubricant.                             Fill to proper level.

2.  Improper lubricant type.                   Replace with proper lubricant.

3.  Excessive trash build-up around        Remove trash.

gearbox.

Blade is scalping ground.           1.  Mower too low.                                 Raise mower, reset tail-wheel and

3-pt. control stop.

2.  Field is ridged.                                   Cut field at different angle.

3. Field is too wet.                                  Stop and wait until it is drier.

Mower will not cut.                     1.  Shear bolt sheared.                            Install new shear bolt.

(shear bolt drive only)

Blade bolts.                                   1. Bolts not tight.                                    Tighten bolts to 350 ft./lb.

2.  Bolt hole elongated or oversized.      Replace blade carrier.

3.  Blade bolt threads are worn               Replace blade bolt, lockwasher,

or damaged.                                        and nut.

Excessive blade wear.                  1.  Cutting in sandy conditions.              Increase cutting height.

2.  Cutting in rocky conditions.               Increase cutting height.

3.  Blades hitting ground.                        Increase cutting height.

# Mower seems to require              1. Advancing into grass or brush             Reduce forward travel speed.

excessive power.                               too rapidly.

2.  Hitting ground.                                    Increase cutting height.

3.  Blades dull or worn.                             Sharpen or replace blades.

4.  Tractor not large enough.                    Use larger horsepower tractor.

Excessive vibration.                     1.  Check gearbox bolts.                           Tighten if loose.

2. Check for loose nuts on                        Tighten if loose.

Blade pan and blades.

3.  Check for bent output shaft. If            Replace shaft if bent.

Shaft is bent, oil will normally

Leak from bottom seal.

4.  Blades locked into position.                Free blades so they swing free.

5.  Check for even wear on each              Weigh each blade. Weight

blade tip. Were both blades                 should be within 1oz. Always

changed at the same time?                   replace both blades.

6.  Broken blade.                                       Replace blades, in set.

7.  Blade pan bent.                                     Replace blade pan.

8.  New blade or bolts matched                 Replace blades or bolts in sets.

With worn blade or bolts.

9.  Drivelines not phased correctly.           Replace driveline.

Implement and tractor yokes

must be in line.

Gearbox noisy                               1.  Rough gears.                                         Run in or change gears.

2.  Worn bearings.                                     Replace bearings.

3. Low oil in gearbox.                               Check level, if low add oil.

Gearbox leaking.                           1. Damaged oil seal.                                  Replace seal.

2.  Bent shaft.                                             Replace oil seal and shaft.

3.  Shaft rough in oil seal area.                   Replace or repair shaft.

4.  Oil seal installed wrong.                        Replace seal.

5. Oil seal not sealing in housing.              Replace seal or use sealant on

outside diameter of seal.

6. Oil level too high.                                   Drain oil to proper level.

7.  Hole in gearbox.                                    Replace gearbox.

8.  Gasket damaged.                                   Replace gasket.

9.  Bolts loose.                                             Tighten bolts.

## LIMITED WARRANTY

Wallace Auto Parts & Services, Inc. warrants to the original purchaser of any new Rotary Mower, that the equipment be free from defects in material and workmanship for a period of ninety (90) days from the date of retail sale.

Replacement or repair parts installed on the equipment covered by this warranty are warranted for ninety (90) days from the date of purchase of such part or to the expiration of the applicable new equipment warranty period, which ever occurs later.

Such parts shall be provided at no cost to the original owner during regular working hours. Wallace Auto Parts & Services, Inc. reserves the right to inspect any equipment or parts which are claimed to have been defective in material and workmanship. This warranty applies only to the repair or replacement of said part, solely at the manufacturer’s discretion.

DISCLAIMER OF IMPLIED WARRANTIES & CONSEQUENTIAL DAMAGES

Wallace Auto Parts & Services’ obligation under this warranty, to the extent allowed by law, is in lieu of all warranties, implied or expressed, including implied warranties of merchantability and fitness for a particular purpose and any liability for incidental and consequential damages with respect to the sale or use of the items warranted. Such incidental and consequential damages shall include but not be limited to: transportation charges other than normal freight charges; cost of installation other than cost approved by Wallace Auto Parts & Services, Inc.; duty; taxes; charges for normal service or adjustments; loss of crops or any other loss of income; rental of substitute equipment, expenses due to loss, damage, detention or delay in delivery of equipment or parts resulting from acts beyond the control of Wallace Auto Parts & Services, Inc.

THIS WARRANTY SHALL NOT APPLY:

- To vendor items which carry their own warranties, such as hydraulic cylinders, and tires.

- If the unit has been subjected to misapplication, abuse, misuse, negligence, fire of other accident.

- If parts not made or supplied by Wallace Auto Parts & Services, Inc. have been used in connection with the unit, if, in sole judgement of Wallace Auto Parts & Services, Inc. such affects its performance, stability, or reliability.

- If the unit has been altered or repaired outside an authorized Wallace Auto Parts & Services, Inc. dealership in a manner which, in the sole judgement of Wallace Auto Parts & Services, Inc. such affects its performance, stability, or reliability.

- To normal maintenance service and normal replacement items such as gearbox lubricant, hydraulic fluid, worn blades, or to normal deterioration of such things as belts and exterior finish, due to use or sun exposure.

- To expendable or wear items such as teeth, chains, sprockets, belts, springs and other items that in the company’s sole judgement is a wear item.

NO EMPLOYEE OR REPRESENTATIVE OF WALLACE AUTO PARTS & SERVICES, INC. IS AUTHORIZED TO CHANGE THIS WARRANTY IN ANY WAY OR GRANT ANY OTHER WARRANTY UNLESS SUCH CHANGE IS MADE IN WRITING AND SIGNED BY WALLACE’S PRESIDENT, 5605 HIGHWAY 34 NORTH, PO BOX 68, RALEIGH, IL 62977-0068. WALLACE RESERVES THE RIGHT, AT OUR DISCRETION, TO MODIFY, AMEND, OR ALTER THIS WARRANTY POLICY, WITHOUT PRIOR NOTICE, AT ANY TIME.

## Safety precautions

Most accidents occur because of neglect or carelessness.

Avoid needless accidents by following all of the safety precautions listed below.

Machinery should be operated only by those who are responsible and are authorized to do so.

Stop the engine, lower all equipment, lock the brakes, and remove the ignition key before dismounting from the tractor.

Never stand between tractor and implement while tractor is being backed to hitch.

Loose fitting clothing should not be worn, to avoid catching on various parts.

Detach implement in area where children normally do not play.

When performing adjustments or maintenance on an implement, first lower it to the ground or block it securely at a workable height.

Only a qualified operator should be permitted on tractor when in operation; no riders allowed.

Make certain everyone is in the clear before starting tractor or raising or lowering equipment.

Operate the tractor and implement only while in driver’s seat.

Reduce speed when transporting mounted implements to avoid bouncing and momentary loss of steering control.

A heavy load can cause instability of the tractor. Use extreme care during road travel. Slow down on turns and watch out for bumps. Tractor may need front counter-weights to counter-balance the weight of the implement.

Reduce speed on hillsides or curves so there is no danger of tipping.

Avoid driving too close to the edge of ditches or creeks.

Do not transport implement on public roads without reflectors and slow moving vehicle emblem in daylight and with approved warning lights at night and other periods of poor visibility.

Due to the width of some implements, use extra caution on highways, farm roads, and when approaching gates.

Always be sure the implement is in the proper position for transport.

Keep alert and watch the front as well as the rear when working with the implement.

KEEP CHILDREN AWAY FROM DANGER ALL DAY, EVERY DAY.

EQUIP TRACTORS WITH ROLLOVER PROTECTION (ROPS) AND KEEP ALL MACHINERY GUARDS IN PLACE.

PLEASE WORK, DRIVE, PLAY, AND LIVE EACH DAY WITH CARE AND CONCERN FOR YOUR SAFETY AND THAT OF YOUR FAMILY AND FELLOW CITIZENS.